Reminders: Disconnect 9V battery when not in use. The battery drains quickly, and the more it drains, the slower your rover

| UCI Life Day 1 | Software Day 2 | Hardware Day 3 | Wiring & Testing Day 4 | Skills Challenge Day 5 |
| --- | --- | --- | --- | --- |
|  | Pi Introduction | Learn about computer vision code | Complete Wiring (See Diagram) | Learn about optimization parameters |
|  | Test Motors with 9V | HSV Values (Learn how to click the colors) | Test Wiring | Run Tournament |
|  | Test Cameras |  | Ensure Motors Have Proper Directionality |  |
|  | Learn about SSH/VNC | Fabricate the Hardware |  |  |

By: AJ Calinisan acalinis@uci.edu

- Software

  - Pi introduction (Ernest’s slides)

    - Give out SD Cards with OS

    - Connect the Rpi to monitor

    - Connect Mouse and Keyboard

    - Rpi Setup

      - Setup user and pass

      - Connect to wifi

    - Frontload some terminal information

      - Explain how to use terminal (shortcuts?)

      - `sudo apt update -y`

  - Think about how the Robots will function. Two components:

    - Motors

    - Cameras

    - How do these work together to play soccer?

  - Test Motors

    - Use direct connection to 9V Battery

    - Ensure motors go backwards and forwards

  - ` sudo apt install python3-opencv python3-gpiozero python3-picamera2 -y`

  - Frontload information about how and why to use SSH/VNC

  - Test Camera

    - Connect the Camera (be careful)

    - Test with `rpicam-hello`

- Computer Vision, HSV, and Hardware

  - Test the Code

    - `sudo git clone https://rasp@github.com/ajcalinisan/aisoccerbot`

    - `cd aisoccerbot`

    - `python green_ball_tracker.py` (Requires Step I.d to be complete)

    - See if it can track a pickleball

  - Learn about HSV values and how opencv tracks colors

    - HSV color picker

    - Click the window in green_ball_tracker.py to see real-life values

  - Fabricate the base (See separate “SoccerBot Assembly” document)

- Wiring and Testing

  - Complete Wiring (See Wiring Diagram)

  - Test

    - Ensure motor_test.py has correct GPIO pins defined (by diagram:

      - right_motor  = Motor(forward=17, backward=18)

      - left_motor = Motor(forward=22, backward=23)

    - Use motor_test.py to confirm forward, back, left, and right movements

    - Check to make sure camera still works

  - Run soccer_bots_no_preview.py on startup. Go to crontab section in README.md

  - Test Outside

    - Ensure that soccerbot works

    - Ensure soccerbot travels straight after it centers the ball in its camera

    - Modify soccer_bot.py

      - `sudo nano soccer_bot.py`

      - `sudo thonny soccer_bot.py`

      - Change these values especially RIGHT_SPEED and LEFT_SPEED (these speed values will likely both have to be 1.0 and 1.0 (not 1.0 and 0.7)

- Tournament Day

  - Further optimizations may be done by tweaking the SPEED, TIME, SLEEP, CENTER, and RADIUS parameters.

  - Run a bracket by timing each robot to see how long it takes them to push a ball out of a designated zone.